Study of Five level Diode Clamped Inverter Fed 3 Phase Induction Motor Drive system for various parameters.

Mr. Ravindra L.Chakole

PG Student ,Department of Electrical Engineering,

Priyadarshini College of Engineering

Nagpur, India

ravindrachakole50@gmail.com

Prof.U.E.Hiwase,

Department of Electrical Engineering

Priyadarshini College of Engineering,

Nagpur , India (440019)

targetumesh28@gmail.com

Abstract—The main aim of this study is to find out the harmonics effect of the source on the performance of the induction motor with different accepts for various application.

If the current or voltage waveforms of power source are distorted from its ideal form it will be termed as harmonic distortion. This harmonic distortion could result because of many reasons.

Keywords—Switching;current harmonics;distortion.

Introduction

The electrical drives is a combination  of an electric motor and

its electric supply system. Thus analysis cannot always be limited to the electric drives but have to be extended over the

whole electric drive from the shaft-end to the electric input power supply.

The main aim of this study is to explain the effect of harmonics distortion of the input power fed through the multilevel inverter  related to the  deviation in motor torque torsional vibrations, etc.

Problem Statement

The output voltage in a conventional two level inverters are  square wave in nature and it contains number of  odd harmonics, which  causes various losses in the induction motor and affects the performance. Thus by increasing the number of levels , Total Harmonic Distortion and losses in the Induction Motor can be reduced”.

Couses of harmonics

Various industrial loads including static converters (such as electric furnace, induction heating devices and switching power supply) inject current harmonics in power systems. Generally power electronic devices such as switching sources and converters are most important sources of harmonic generation. Converters usually generate harmonics from nth level in AC side, given by

n = k np +/- 1

Where:

k - is a constant and

np is the number of converter pulses.

Effect of Inverter Harmonic

1. Third Harmonics  : Third harmonic flux waves produced by each of the three phases neutralize each other as it differs in time phase by 120o. Thus air gap flux does not contain third harmonics and its multiplies. The fundamental mmf wave produces flux which rotates at synchronous speed which given as ns = 120 f1/P rpm where f1 is supply frequency and P is number of poles.

2. Fifth Harmonics : Fifth harmonic mmf wave produces flux which rotates at   120 f1/5P = ns/5 rpm and in direction opposite to the fundamental mmf wave.

3. Seventh Harmonics : Similarly , seven harmonic mmf produces flux which rotates at ns/7 rpm and in the direction of fundamental m.m.f. wave.

Thus it can be seen that harmonic m.m.f. wave produces flux which rotates at 1/K times the fundamental speed and in the direction of fundamental wave if K = 6m + 1 and in the reversed direction if K = 6m - 1 where m is any integer. The most important and predominant harmonics whose effects must be minimized are 5th and 7thharmonics.

Induction Motor Parameter:

| Sr No. | Parameter | Values in unit. |
| --- | --- | --- |
| 1 | Stator Voltage | 415 Volt. |
| 2 | Stator steady state Current | 7.9 Amp |
| 3 | Rated Speed | 1460 rpm |
| 4 | Load Torque | 25.08 N-m |

Table :1

Induction Motor Parameter calculated for simulation .

| Sr No. | Parameter | Values in unit. |
| --- | --- | --- |
| 1 | Rotor Current | 3.95 – 4.0 A |
| 2 | Stator Resistance | 1.9- 2.4 Ohms |
| 3 | Rotor Resistance | 1.8- 1.94 Ohms |
| 4 | Stator core loss current | 6.356 A |
| 5 | Magnetizing Current | 4.74 A |
| 6 | Excitation Current | 2.72 A |
| 7 | Stator core loss Resistance | 37.72 Ohms |
| 8 | Stator Magnetizing Inductor | 0.1542 H |
| 9 | Magnetizing Reactance | 48.44 Ohms |
| 10 | Slip | 5.30 % |
| 11 | Rotor Frequency | 2.65 Hz |

Table :2

Simulation Result of Two level inverter

- Two Level Inverter

With above parameter and rating of Induction motor, a MATLAB simulation is done for two level inverter with input DC voltage of 770 V, Modulation Index of 0.9 and switching frequency of 5khz. Following results are obtain for line current and voltage.

Fig : 1 Line current ( Two Level )

Fig 2 – THD analysis ( line current -  Two Level)

Fig : 3 Line voltage ( Two Level )

Fig 4 – THD analysis ( line voltage -  Two Level)

Fig 5 – Speed Deviation ( Two Level)

Fig 6 – Torque Deviation ( Two Level)

- Three o Level Inverter:

With the same parameter of motor and DC voltage source, a simulation is performed for Three level VSI Inverter and following results are obtain for THD analysis of line current and voltage.

Fig : 7 Line current ( Three Level )

Fig 8 – THD analysis ( line current -  Three Level)

Fig : 9 Line voltage ( Three Level )

Fig 10 – THD analysis ( line voltage -  Three Level)

Fig 11 – Speed Deviation ( Three Level)

Fig 11 – Torque  Deviation ( Three Level)

- THD Comparison of Two Level & Multilevel Inverter ( Three Level)

| Parameter | Two Level | Three Level | % Change |
| --- | --- | --- | --- |
| Current THD | 10.01 % | 5.05 % | 50.44 % |
| Voltage THD | 85.38 % | 43.48 % | 50.92 % |
| Speed Deviation | 1425- 1460 rpm | 1435-1460 rpm |  |
| Torque Deviation | 10-40 Nm | 15-35 Nm |  |

Table : 3

Conclusion : From the above comparison table , it is seen that the input power quality of the inverter can be improved by increasing the levels of inverter , also the deviation in the mechanical parameter can also improve.

References :

1] Mohan M.Renge and Hiralal M.Suryawanshi , “Five –Level Diode Clamped Inverter to Eliminate Common Mode Voltage and Reduce dv/dt in Medium Voltage Rating Induction Motor”,IEEE Transactions on Power Eletronics , Vol. 23, No. 4 July 2008, pp 1598-1606.

2] Prasad, H.; Sudhakar, T.D.; Chilambarasan, M., "Mitigation of current harmonics in a solar hybrid system by installation of passive harmonic filters," in International conference on Computation of Power, Energy Information and Commuincation (ICCPEIC),  pp.0345-0349, 22-23 April 2015

3] Shashikant R.Vaishnani, Hardik H.Raval , Bhavans N.Shah, “Mitigation of Harmonics in A.C.Drive by using multi-pulse Transformer Solution.” International Journal of Engineering Science and Innovative Technology ( IJESIT) , Vol., 2 , Issue 2 , March 2013, pp – 653-662.

4] Davari, P.; Yongheng Yang; Zare, F.; Blaabjerg, F., "A novel harmonic elimination approach in three-phase multi-motor drives," in  IEEE conference on Energy Conversion Congress and Exposition , pp.7001-7008, 20-24 Sept. 2015.

5] Lee, K.; Carnovale, D.; Young, D.; Ouellette, D.; Zhou, J., "System harmonic interaction between DC and AC adjustable speed drives and cost effective mitigation," in IEEE conference on Energy Conversion Congress and Exposition (ECCE),  pp.4069-4076, 20-24 Sept. 2015

6] Concettina Bucccella, Carlo Cecati, Maria Gabriella Cimoroni and Kaveh Razzi, “ Harmonic Mitigation Technique for Multilevel Inverter in Power Systems,” International Symposium on Power Electronics , Electrical Drives , Automation and Motion,  pp – 73-77, 2014

7] Edirs Pouresmaeil , Danial Montesinos-Miracle and Orioi Bellmunt, “ Control Scheme of Three –Level NPC Inverter for Integration of Renewable Energy Resources into AC Grid.” IEEE Transaction  System Journal , Vol- 6 ,No. 2 , pp 242-253, June 2012.

8]  Sule Ozdemir, Engin Ozdemir , Leon M.Tolbert and Surin Khomfoi , “ Elimination of Harmonics in a Five –Level Diode Clamped Multilevel Inverter Using Fundamental Modulation.” International Conference on Power Electronics and Drive Systems ,Bangkok, Thailand, Pp 850-854, 27-30Nov. 2007.

9] Amirhossein Moeini, Hossein Iman-Eini, Mohanadkasem Bakhshizadeh. “ Selective harmonic mitigation –pulse-width modulation technique with variable DC-link voltages in single and three –phase cascaded H-bridge inverters.” IET Transaction on Power Electronics,  Vol- 7 , Issue- 4 , pp – 924-932,2014

10] Ankita Papriwal, Dr. Amita Mahar, “ Mitigation of Harmonics in Inverter.” IOSR Journal of Engineering ( IOSRJEN) , Vol – 2 , Issue – 9 , pp- 98-105, Sept – 2012 .

11] Sharana Reddy, B. Basavaraja , “ Simulation and Analysis of Common Mode Voltage in 2 Level and multilevel Inverter Fed Induction Motor Drive with Long Cable.” International Journal of Engineering and Advanced Technology ( IJEAT), Vol- 2 , Issue- 2, PP- 406-410, Dec-2012 .

12]  S.jaisiva,  sSneelan,  T.ilansezhian.”  Harmonic analysis in      non – linear loads of power system,International Research Journal of Engineering and Technology (IRJET), Volume: 03 Issue: 05  May-    2016, PP1474-1478.