Smart Technologies for Air Pollution Monitoring System Using WSN

Ms. Harana M. Bodele1, Dr.G.M.Asutkar2

1Research scholar PIET ,Nagpur ,India,440019

2 Dr. G.M. AsutkarV. PrincipalPIET, Nagpur, India,440019

Abstract – Today's major public and environmental issues are air pollution and traffic congestion. The  first Air quality joint inter comparison exercise was organised in Aveiro (Portugal) which was focussed on the evaluation and assessment of the environmental gas , particulate matter (PM) and meteorological micro sensors versus standard air quality reference methods. In this paper the review is studied on different  hardware and software techniques used for the air pollution monitoring methods  using WSN. Due to the advancement in wireless communication wireless sensor networks (WSN)are used in different areas. Here it is focussed basely on the low energy consumption of the PM sensor and other sensors. The parameters which pollute the air are mainly black carbon, particle number concentration, alveolar lung-deposited surface area, mean particle diameter,PM2.10,PM2.5,PM1. Low-cost sensor technology can potentially revolutionise the area of air pollution monitoring and can  provide high density spatiotemporal pollution data. Also mathematical and statistical technique for sensor calibration ,fault detection and data quality assurance can help for the low cast air pollution sensor network.

INTRODUCTION

Wireless Sensor Network is a part of Wireless Network. Wireless Network stands for defining a technology in which two or more systems communicate with each other without wired connection. Wireless  Sensor Network consists of many sensors that work to gather information from the real world. sensor need to carry energy in their pocket. A WSN consists of a collection of sensor nodes and a base station (BS) connected through wireless channels with sensing, wireless communications and computational capabilities. The advantages of WSN are ease of deployment, low installation cost, distribution over a wide region and high fault tolerance. The node

deployment option affects the performance of routing protocol in terms of energy consumptions. There are three  ways in which tiny sensor nodes can be deployed in a WSN environment: Regular Deployment-Sensor nodes can be deployed in a fixed manner and data is routed through a predefined path. once sensor nodes are deployed, WSN performed a specific job. In WSN, energy is a big challenge because nodes are powered by batteries. This energy can be very expensive, difficult or even impossible to renew. So saving energy to maximize network lifetime is one  of the critical problems in WSN. In WSN, nodes dissipate energy in processing and transmitting/receiving messages. hierarchical routing or cluster based routing is to efficiently maintain the energy usage of sensor nodes by involving them in multi-hop communication within a particular cluster. The first hierarchical protocol is the Low Energy Adaptive Clustering Hierarchy (LEACH) that was introduced in .Many hierarchical protocols were emerged from LEACH. Degraded air quality is an environmental and health issue affecting citizens around the globe. Although there is significant research and development of microsensors for applications in pollutant monitoring, the interpretation of sensor signals from field campaigns remains limited and challenging.[18]

METHODOLOGY and  CONCLUSION

Murat Dener et al. The WSN  and Smart other  technologies are reviewed in Smart Environment to monitor weather pollution ,radiation levels,electromagnetic levels, advanced lighting systems, noise mapping within the city and waste  management. It consist of good agricultural production, environmental protection and new data generation . In the smart grids, it reduced  the power and  enhanced power management system which monitored  and controled the balance between the transfer-distribution and the consumers.[20]. Hirofumi Ohyama et al. retrived   lower-tropospheric O3 data from the TANSO-FTS .And TIR spectra were validated with those derived from a tropospheric O3 differential absorption lidar (DIAL) located at Saga, Japan [19],here the DIAL system measured the profiles of O3 from 1 km  to 10 km.  in day time and up to 20 km in night . then comparison done  between the TANSO-FTS and DIAL data, The atmospheric CO column measurements is  taken  with a ground-based high-resolution FTS within 12 hours. To ensure accurate comparison the DIAL, O3 profiles were smoothed by the TANSO-FTS averaging kernels . The XCO data were calibrated with in situ data from aircraft observations above several TCCON sites, and the uncertainty (2s) of the XCO data was estimated to be 4 ppb [19], The LTOC and the XCO data was retrived from the TIR spectra measured by the TANSO-FTS onboard the GOSAT during the period from April 2009 to July 2015 and from the ground-based high-resolution FTS spectra during the period from July 2011 to July 2015. The TANSO-FTS LTOCs were compared with those derived from the O3 DIAL the mean difference was _2.24 ± 1.77 DU. mean values of the LTOCs had maximums in June and local minimums in July and November. The monthly mean values of the XCO indicated a broad spring maximum and a broad summer autumn minimum. C. Borrego et al.[19] .  The performance of three portable monitors (micro-aethalometer AE51, DiscMini, Dusttrak DRX) was assessed [18] They evaluated  the parameters like  black carbon, particle number concentration, alveolar lung-deposited surface area, mean particle diameter, PM10, PM2.5 and PM1. and the comparison was done to test the performance with stationary instruments (MAAP, CPC, SMPS, NSAM, GRIMM aerosol spectrometer). Results was good in  most portable and stationary instruments, with R2 values having mostly >0.80. Relative difference between portable and stationary instruments was mostly <20%, and <10% between different units of the same instrument. The  cluster of ozone, nitrogen dioxide, nitrogen monoxide, carbon monoxide and carbon dioxide sensors was used. For this the sensor used were  of metal oxide or electrochemical .[17] They studied wheather  the sensors could reach the Data Quality Objective (DQOs) of the European Air Quality Directive for indicative methods (between 25 and 30% of uncertainty for O3  and NO2 . it was found that O3 can  be calibrated using simple regression techniques while NO2 a need  better agreement between sensors and reference measurements.  Here the work accessed wheather carbon monoxide sensors can reach the Data Quality Objective (DQOs) of 25% of uncertainty set in the European Air Quality Directive for indicative methods.  Result was found for NO, CO and CO2 that the best agreement between sensors and reference measurements  was observed for learning techniques compared to linear and multilinear regression.[9] . Here when  operated suitably, CO, NO and NO2  the electrochemical sensors can provide parts-per-billion level mixing ratio sensitivity with low noise and high linearity,making them suitable for urban air quality measurements.[8].  The small, portable gaseous air pollution monitor  were used and technologies identified .Then online databases was took for articles containing specific information related to performance, characteristics, and use of the monitors that measure one or more of three criteria gaseous air pollutants: ozone, nitrogen dioxide, and carbon monoxide was studied and  comparison between their applications,physical features, sensing capabilities, and costs of the devices done.  result found that recent portable monitoring trends are strongly related to associated applications and audiences.. Xiaobing Pang et al. Ozone sensors are used for O3 detection ,these  are compact in size, low cost, low power and fast response. Generally  O3 sensors use either electrochemical sensors  or are based on semiconductor O3 sensors. the performance of electrochemical O3 sensors were investigated in laboratory and ambient air quality monitoring. The result  indicated that the miniaturised O3 sensor was a suitable alternative for O3 measurements in both laboratory experiment and air-quality monitoring.[16] The idea of  employing sensors in a small space is  a common attention from the air quality management community.  Effort of scientists and instrument manufacturers have made it possible to reduce the cost of air pollution sensors. Hence at present, the manufacturing  cost of these systems does not go high. But technological challenges regarding the use of sensors for air pollution monitoring is to improve their sensitivity, stability and longevity of operation before replacement. Most low cost air pollution sensors are sensitive down to a few hundreds of ppb.  but  most of the important pollutants concentrations are below this limit, there is a pressing tech  to lower these threshold limits . The large amount of data expected to be collected by the sensors is not yet widely available or affordable .  The large amount of data  is collected by wireless sensor networks has to be routed to a single managing entity and so  the network sink, hence the algorithms for data fusion and aggregation  are needed to reduce congestion and system overloading .[11 ] The commercial low-cost sensors are  promising, and  results showed  that some sensors, i.e., NO and PM10, are already capable to offer  coarse information about air quality, indicating if the air quality is good ,moderate or if the air is heavily polluted. This type of information can be used for applications which aim to raise the awareness,[12  ] AIR quality is monitored using a microcontroller and WSN measuring two main gases: Carbone Monoxide (CO) and Liquid Petroleum Gas (LPG). A sample of obtained results from both clean environment and KFU library are shown in table 1 .

Table 1. Clean Air Vs. KFU Library :

Clean Air                             KFU    Library

LPG 	         CO                  LPG		 CO

2.06                0.03		 80.77		 1.63

4.41                0.02 		85.92 		1.87

3.49               0.02 		76.59 		2.7

2.56 	      0.04 	              54.54 		4.45

2.2         	     0.05 		49.81		 0.68

2.03   	     0.03 		52.55 		3.65

2.12              0.02		 78.1		 0.98

2.12             0.03		68.21 		5.94

3.4	    0.02		 61.71		 5.03

4.51 	    0.02	               46.51 		1.45

2.21             0.06 		96.09		 6.04

3.13 	     0.02 		75.38 		5.92

2.63 	    0.03 		102.3    		4.13

the results showed that there was a big difference in the levels of both gases (LPG and CO) which obtained from the several tests and circuit runs.The acquired results show no risky situation to be considered for further action. WSN provides a real-time information about the level of air pollution in different area also it provide alerts in cases of drastic change in quality of air. This design can be enhanced by adding a wireless network card to the microcontroller circuitry for better and easier control of the sensors readings. Also the circuit can be improved to measure the level of other gases in the air such as Sulfureted Hydrogen (H2S),Ammonia (NH3), Alcohol and many others.[13]  Testing the long term stability and the effectiveness of the  routing algorithm,  several nodes were placed for local temperature and humidity monitoring in wine yards. The downy mildew fungi plasmopara  viticola is one of the most problematic infections in wine yards and can conclude in a total loss of wine grapes. The sensor

network can help the farmers to detect the regions in which the growth of downy mildew has high probability and makes a local application of fungicides necessary.[14]

REFERENCES

- SONAL O TALOKAR, MANJUSHA DESHMUKH.  [2013] WSN FOR AIR POLLUTION MONITORING SYSTEM.. International Journal of Electrical, Electronics and Data Communication,  Volume-1, Issue-10, Dec-2013

- Prof.S.M.Bankar1, Abhijeet Deshmukh2, Shrirang Shewale3, Ashvini Deshmukh [2016] .  Pollution Monitoring & Intelligent Information Exchange System for Traffic Congestion. INTERNATIONAL JOURNAL OF INNOVATIVE RESEARCH IN ELECTRICAL, ELECTRONICS, INSTRUMENTATION AND CONTROL ENGINEERING Vol. 4, Issue 3, March 2016.

- K.E. Kelly , J. Whitaker , A. Petty , C. Widmer , A. Dybwad , D. Sleeth , R. Martin , A. Butterfield [2017]. Ambient and laboratory evaluation of a low-cost particulate matter sensor. Environmental Pollution 221 (2017) 491-500

- Sharon Moltchanov, Ilan Levy, Yael Etzion, Uri Lerner, David M. Broday , Barak Fishbain [2015]. On the feasibility of measuring urban air pollution by wireless distributed  sensor networks . Science of the Total Environment 502 (2015) 537–547.

- Georgia Miskell , Jennifer Salmond, David E. Williams [2017]. Low-cost sensors and crowd-sourced data: Observations of siting  impacts on a network of air-quality instruments. Science of the Total Environment 575 (2017) 1119–1129

- Ana Maria Todea , Stefanie Beckmann , Heinz Kaminski , Delphine Bard, Sébastien Bau  Simon Clavaguera ,, Dirk Dahmann, Hélène Dozol , Nico Dziurowitz , Karine Elihn, Martin Fierz , Göran Lidén , Asmus Meyer-Plath , Christian Monz , Volker Neumann , Johannes Pelzer ,Barbara Katrin Simonow, Patrick Thali , Ilse Tuinman, Arjan van der Vleuten ,  Huub Vroomen l, Christof Asbach  [2017] .Inter-comparison of personal monitors for nanoparticles  exposure at workplaces        and in the environment. Science of the Total Environment 605–606 (2017) 929–945

- Aakash C. Rai , Prashant Kumar , Francesco Pilla , Andreas N. Skouloudis , Silvana Di Sabatino , Carlo Ratti , Ansar Yasar , David Rickerby  [2017] .End-user perspective of low-cost sensors for  outdoor air pollution monitoring. Science of the Total Environment 607–608 (2017) 691–705

- Laurent Spinellea,, Michel Gerbolesa,, Maria Gabriella Villanib, Manuel Aleixandrec,Fausto Bonavitacola  [2017]  Field calibration of a cluster of low-cost available sensors for airquality monitoring. Part A: Ozone and nitrogen dioxide . Sensors and Actuators B 215 (2015) 249–257.

- Laurent Spinellea,, Michel Gerbolesa,, Maria Gabriella Villanib, Manuel Aleixandrec,Fausto Bonavitacolad [2017].Field calibration of a cluster of low-cost commercially availablesensors for air quality monitoring. Part B: NO, CO and CO2. Sensors and Actuators B 238 (2017) 706–715.

- Xiaobing Pang, Marvin D. Shaw, Alastair C. Lewis, Lucy J. Carpenter,Tanya Batchellier  [2017] Electrochemical ozone sensors: A miniaturised alternative for ozonemeasurements in laboratory experiments and air-quality monitoring . Sensors and Actuators B 240 (2017) 829–837

- Prashant Kumar , Lidia Morawska , Claudio Martani , George Biskos , Marina Neophytouh,   Silvana Di Sabatino , Margaret Bell , Leslie Norfordk, Rex Britter  [2015].The rise of low-cost  sensing for managing air pollution in cities. Environment International 75 (2015) 199–205.

- Nuria Castell , Franck R. Dauge , Philipp Schneider , Matthias Vogt , Uri Lerner , Barak Fishbain , David Broday , Alena Bartonova  [2017.Can commercial low-cost sensor platforms contribute to air quality monitoring and exposure estimates? Environment International 99 (2017) 293–302

- Qasem Abu Al-Haija, Hasan Al-Qadeeb and Abdulmohsen Al-Lwaimi [2013]. Case Study: Monitoring of AIR quality in King Faisal University using a microcontroller and WSN. ScienceDirect, Procedia Computer Science 21 ( 2013 ) 517 – 521

- Thomas Posnicek, Karlheinz Kellner, Martin Brandl [2014] .Wireless Sensor Network for Environmental Monitoring with 3G Connectivity. ScienceDirect ,Procedia Engineering 87 ( 2014 ) 524 – 527

- Grant R. McKercher , Jennifer A. Salmond b, Jennifer K. Vanos  [2017] . Characteristics and applications of small, portable gaseous air pollution monitors . Environmental Pollution 223 (2017) 102e110

- M.I. Mead , O.A.M. Popoola , G.B. Stewart , P. Landshoff , M. Calleja , M. Hayes, J.J. Baldovi , M.W. McLeod , T.F. Hodgson , J. Dicks , A. Lewis , J. Cohen , R. Baron , J.R. Saffell , R.L. Jones [2013]. The use of electrochemical sensors for monitoring urban air quality in low-cost, high-density networks. Atmospheric Environment 70 (2013) 186-203

- M. Viana , I. Rivas , C. Reche , A.S. Fonseca , N. P_erez , X. Querol , A. Alastuey ,  M. _Alvarez-Pedrerol , J. Sunyer [2015] . Field comparison of portable and stationary instruments  for outdoor urban air exposure assessments . Atmospheric Environment 123 (2015) 220e228

- C. Borrego , A.M. Costa , J. Ginja , M. Amorim , M. Coutinho , K. Karatzas , Th. Sioumis , N. Katsifarakis , K. Konstantinidis , S. De Vito , E. Esposito , P. Smith ,  N. Andr_e , P. G_erard , L.A. Francis , N. Castell , P. Schneider , M. Viana ,  M.C. Minguill_on , W. Reimringer , R.P. Otjes , O. von Sicard , R. Pohle , B. Elen ,  D. Suriano , V. Pfister , M. Prato , S. Dipinto , M. Penza.[2016].  Assessment of air quality microsensors versus reference methods: The EuNetAir joint  exercise . Atmospheric Environment 147 (2016) 246e263

- Hirofumi Ohyama , Shuji Kawakami , Osamu Uchino , Tetsu Sakai , Isamu Morino ,Tomohiro Nagai , Kei Shiomi , Masanori Sakashita , Taiga Akaho , Hiroshi Okumura ,Kohei Arai  [2016].Seasonal variation of the O3eCO correlation derived from remotesensing measurements over western Japan. Atmospheric Environment 147 (2016) 344e354

- Murat Dener,Cevat Bostancioglu,[2015] . Smart Technologies with Wireless Sensor Networks. ScienceDirect, Procedia - Social and Behavioral Sciences 195 ( 2015 ) 1915 – 1921